|  |  | # REPORT |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ## AUTOMATED TELLER MACHINE USING FSM CONTROLLER |  |  |  |  |
|  |  |  |  | ### Date ### 14/07/2023 |  |  |  |  |

BLOCK DIAGRAM:

FSM MODEL:

Next state model

APPROACH TO SOLVE THE PROBLEM:

To solve the problem described in the modatm module, the following approach was taken:

- Declare States: The module declares several states using parameter assignments. These states represent different stages of the ATM operation, including IDLE, CARD_INSERTED, PIN_ENTERED, WITHDRAWAL, DEPOSIT, and BALANCE_INQUIRY.

- Declare Registers: The module defines several registers to store important information. current_state and next_state are used to keep track of the current and next states in the FSM. account_balance stores the current balance amount, and selected_language holds the selected language option.

- Moore Outputs: The module assigns values to the output signals based on the current state and input conditions. The cash signal indicates whether cash should be dispensed during a withdrawal, and deposit_complete signals if a deposit transaction has been completed. balance represents the current account balance, and language represents the selected language.

- FSM Logic: The module uses an always block triggered by the positive edge of the clock or reset signal to handle the FSM logic. When the reset signal is asserted, the module initializes the necessary variables. Otherwise, it updates the current state based on the next state assignment.

- State Transitions: The FSM transitions between different states based on the current state and input conditions. Each case in the case statement represents a specific state, and the input conditions determine the next state. For example, if the current state is IDLE and the card_in signal is asserted, the next state becomes CARD_INSERTED.

- Output Updates: The module updates the output signals based on the current state and input conditions. The ready signal indicates if the ATM is ready for a new transaction, and error signals if an error occurred during a withdrawal. The account balance is updated by adding the deposit amount if a deposit request is made. The selected language is updated based on the language selection signal.

By following this approach, the modatm module implements the logic of an ATM controller, allowing it to handle card insertion, PIN entry, withdrawal, deposit, balance inquiry, and language selection functionalities.

COMPLETE FLOW TO  SOLVE THE PROBLEM:

To solve the problem described in the modatm module, the following complete flow was followed:

- Initialization:

  - When the reset signal is asserted, the module initializes the necessary variables.

  - current_state is set to IDLE.

  - ready and error signals are set to 0.

  - account_balance is set to 0.

  - selected_language is set to 00.

- Clock and Reset Handling:

  - The module is triggered by the positive edge of the clk signal or the positive edge of the reset signal.

  - When the reset signal is asserted, the module goes back to the initialization step.

- State Transitions:

  - The module uses a case statement to determine the next state based on the current state and input conditions.

  - If the current state is IDLE and the card_in signal is asserted, the next state becomes CARD_INSERTED.

  - If the current state is CARD_INSERTED and the pin_entry signal is asserted, the next state becomes PIN_ENTERED.

  - If the current state is CARD_INSERTED and the balance_inquiry signal is asserted, the next state becomes BALANCE_INQUIRY.

  - If the current state is CARD_INSERTED and the withdrawal signal is asserted, the next state becomes WITHDRAWAL.

  - If the current state is CARD_INSERTED and the deposit signal is asserted, the next state becomes DEPOSIT.

  - If the current state is PIN_ENTERED and the balance_inquiry signal is asserted, the next state becomes BALANCE_INQUIRY.

  - If the current state is PIN_ENTERED and the withdrawal signal is asserted, the next state becomes WITHDRAWAL.

  - If the current state is PIN_ENTERED and the deposit signal is asserted, the next state becomes DEPOSIT.

  - If the current state is WITHDRAWAL, DEPOSIT, or BALANCE_INQUIRY, the next state becomes IDLE.

- Output Updates:

  - The module updates the output signals based on the current state and input conditions.

  - The ready signal is set to 1 when the current state is IDLE, indicating that the ATM is ready for a new transaction.

  - The error signal is set to 1 when the current state is WITHDRAWAL and the withdrawal signal is asserted, indicating an error during withdrawal.

  - The cash signal is assigned based on the conditions: cash = (current_state == WITHDRAWAL && withdrawal).

  - The deposit_complete signal is assigned based on the conditions: deposit_complete = (current_state == DEPOSIT && deposit).

  - The balance signal is assigned the value of account_balance.

  - The language signal is assigned the value of selected_language.

- Account Balance Update:

  - If the deposit signal is asserted, the module adds the deposit value to the account_balance variable.

- Selected Language Update:

  - If the language_select signal is asserted, the module updates the selected_language variable with the value of language_select.

FILES UPLOADED IN GIT HUB:

- Model

- code with test bench

RESULT:

The modatm module is a Verilog implementation of an ATM controller. It operates as a finite state machine (FSM) and handles various ATM operations. The module takes inputs such as clock signals, reset signals, and user commands like card insertion, PIN entry, withdrawal, deposit, balance inquiry, and language selection. It provides outputs including ready and error signals, as well as signals for cash dispensing, deposit completion, balance amount, and selected language.

The module handles updating the account balance when a deposit request is made and allows for language selection. It tracks the readiness of the ATM, indicating whether it is prepared for user input, and handles error conditions during withdrawal operations. Overall, the module provides the necessary functionality to control an ATM and perform common transactions.